Using Blank Ammo Training-Ammunition 9mm UZI SMG

General

- Blank Ammo allows shooting the UZI SMG with similar conditions to shooting live ammunition. This ammunition is similar to wood-slugs/bullets.

- This ammunition will be essentially used for: a) two-positional exercises, b) to stop an enemy – in exercises and areas where live ammunition is not allowed.

Barrel Description

- The external shape of the barrel for the 9mm blank ammo was altered from the regular barrel of the UZI SMG; and here are the changes:

  - In the external part of the barrel, which stands out from the SMG’s body, 6 grooves.

  - There is an area on the external side of the barrel, which reads: “Blank firing ammo only.”

  - In the barrel’s muzzle has a thread; for screwing the muzzle-strait

  - The muzzle-strait has a rectangular shape, and the bottom part has a thread. Inside there is an area that has 4 holes/punctures so the gas will come out of them.

  - At the top of the strait, externally, there are 4 knobs, and with them it is possible to take apart or assemble the strait (with a coin, screw driver, etc.)

Bullet Description

- The 9mm Blank firing bullet has a lengthened cartridge; its head is pressed in 3 spots in the shape of a slug/bullet. It’s colored blue or purple on its upper edge.

Operation Method

- In order to shoot the 9mm blank firing ammunition in the UZI SMG, it is inevitable to use a special barrel (as described above); you can shoot semi and burst.

- To change the regular barrel with the special barrel,  press on the pin-screw-nut-barrel clutch, remove the screw-nut and pull out the barrel; afterwards insert the special barrel and tighten it using the screwnut-barrel clutch. To disassemble do the steps backwards.

- Make sure that the muzzle-strait will be screwed on the barrel all the way.

- The special barrel operates with the same principle as the muzzle-strait. In the strait there are 4 small holes, to let some of the gas out, and at the same time there is enough gas pressure created to cock the SMG again.

- Do not shoot blank-firing ammo in regular barrels – because it could cause damage to the barrel.

Care & Cleaning

- Care and clean the special barrel the same way you would clean the regular barrel. Furthermore, after shooting clean the soot from the muzzle-strait, and make sure that the 4 holes will be clean and not sealed.

Safety Instructions

- Do not shoot blank ammunition towards soldiers in a range shorter than 10 meters.

- Under no circumstances, do not shoot live ammo in an SMG which has the special barrel for the blank-firing ammo.

- When loading or unloading, make sure the barrel is pointed up at an angle of 60 degrees, like described in lesson 4.

- Before firing blank ammunition, the commander who is responsible for the exercise, will make sure that the soldiers have no other bullets.

- Commanders or soldiers that have live ammunition (for security purposes) will not receive blank-firing ammunition and they will not participate in the shooting drill/exercise.